SPECIFICATION DOCUMENT

RISC-V SoC with Hardware-Accelerated CNN-Based Image Similarity and Anomaly Detection

Implementation-ready specification for Kiro / RTL development

# 0. Document Purpose

This document converts the current project architecture into an implementation-oriented specification that can be supplied to Kiro for RTL generation and integration. It defines the system architecture, control path, data path, 2-master × 8-slave memory-mapped interconnect, memory map, registers, signals, errors, interrupts, operation sequence, and verification requirements.

Important architecture terminology: the conceptual system bus is specified as a 32-bit memory-mapped bus. The faculty-provided interconnect generator may be used as the RTL implementation mechanism; this document does not redefine the conceptual bus as AXI unless the implementation interface requires it.

# 1. Introduction

## 1.1 Project Title

Design of a RISC-V SoC with a Hardware-Accelerated CNN-Based Image Similarity and Anomaly Detection Engine

## 1.2 System Description

The proposed project implements a RISC-V based System-on-Chip for embedded image similarity and anomaly detection. A small grayscale image is processed by a lightweight CNN accelerator to generate a compact embedding. The generated embedding is compared against stored reference embeddings. The distance/anomaly engine calculates the distance to each reference, selects the minimum distance, and compares it against a programmable threshold to classify the input as MATCH or ANOMALY.

The RISC-V processor is the system controller. Three project-specific IPs are integrated: (1) CNN Feature Extraction Accelerator, (2) Distance/Anomaly Scoring Engine, and (3) DMA Controller. The SoC also contains faculty-provided RISC-V, IMEM, DMEM, UART, Timer and GPIO.

## 1.3 Problem Statement

Software-only CNN inference on a general-purpose processor requires many multiply-accumulate operations and can increase latency and CPU utilization. Dedicated hardware is therefore used for CNN feature extraction and distance scoring while the RISC-V performs configuration, sequencing and result management.

## 1.4 Objectives

Integrate the faculty-provided RISC-V processor into a complete SoC.

Provide memory-mapped access to all peripherals and project IPs.

Implement a lightweight CNN feature-extraction accelerator.

Support INT8 image and weight processing.

Generate a compact embedding vector from the input image.

Compare the query embedding with multiple reference embeddings.

Support a configurable distance metric and anomaly threshold.

Implement DMA-based movement of image, weight and embedding data.

Provide interrupt-based completion notification.

Report the final classification through software/UART and optionally GPIO.

Verify hardware results against a Python/NumPy golden model.

# 2. Top-Level Architecture

The top-level functional blocks are:

RISC-V Core — primary system master; configures IP registers, starts operations, polls status and handles interrupts.

32-bit Memory-Mapped System Bus / Interconnect — routes transactions according to the memory map.

DMA Controller — secondary bus master for bulk data movement and a memory-mapped slave for CPU configuration.

CNN Accelerator — performs input buffering, INT8 convolution/MAC processing, pooling and embedding generation.

Distance/Anomaly Engine — performs query/reference distance calculation, minimum-distance selection and threshold comparison.

IMEM — instruction memory.

DMEM — 64-KB data memory holding input image, CNN weights, reference embeddings and generated/query embeddings.

UART, Timer and GPIO — standard SoC peripherals.

## 2.1 Master / Slave Definition

| Item | Count | Role |
| --- | --- | --- |
| Masters | 2 | M0 RISC-V Core; M1 DMA Controller |
| Addressable slave regions | 8 | IMEM, DMEM, UART, Timer, GPIO, CNN, Distance Engine, DMA registers |
| DMA | Both | Master for data transfers; slave for memory-mapped configuration/status |

## 2.2 Conceptual Interconnect

The interconnect has two incoming master interfaces and eight decoded slave targets. The RISC-V normally initiates configuration and memory transactions. DMA requests bus ownership when it performs bulk transfers. A fixed-priority arbitration policy may be used, with CPU transactions retaining normal priority and DMA obtaining access according to the project arbitration policy.

Conceptual connectivity: RISC-V → Interconnect; DMA master → Interconnect; Interconnect → IMEM/DMEM/UART/Timer/GPIO/CNN/Distance/DMA-register interface.

# 3. Full Memory Map

The following map is the implementation baseline for the 2×8 interconnect. IMEM and DMEM use 64-KB regions. Each peripheral/IP occupies one 4-KB memory-mapped region.

| Port | Slave | Base Address | End Address | Size | Addr Width |
| --- | --- | --- | --- | --- | --- |
| M00 | IMEM | 0x0000_0000 | 0x0000_FFFF | 64 KB | 16 bits |
| M01 | DMEM | 0x1000_0000 | 0x1000_FFFF | 64 KB | 16 bits |
| M02 | UART | 0x2000_0000 | 0x2000_0FFF | 4 KB | 12 bits |
| M03 | Timer | 0x2000_1000 | 0x2000_1FFF | 4 KB | 12 bits |
| M04 | GPIO | 0x2000_2000 | 0x2000_2FFF | 4 KB | 12 bits |
| M05 | CNN Accelerator | 0x2000_3000 | 0x2000_3FFF | 4 KB | 12 bits |
| M06 | Distance/Anomaly Engine | 0x2000_4000 | 0x2000_4FFF | 4 KB | 12 bits |
| M07 | DMA Controller Registers | 0x2000_5000 | 0x2000_5FFF | 4 KB | 12 bits |

## 3.1 Interconnect Generator Configuration

For the faculty-provided Python wrapper, use the 2-master × 8-slave configuration:

python3 axi_interconnect_wrap.py -p 2 8

In the wrapper naming convention, S_COUNT=2 represents the two incoming master-side interfaces, while M_COUNT=8 represents the eight outgoing slave-side interfaces. The project-level naming used in this document is M0/M1 for functional bus masters and M00–M07 for decoded slave targets to avoid ambiguity.

# 4. Control Path

The control path is software-driven. RISC-V writes configuration registers, asserts START bits, monitors BUSY/DONE/ERROR status, and services completion interrupts. Control/status registers are memory mapped through the system bus.

| Source | Control/Status | Destination |
| --- | --- | --- |
| RISC-V | configuration writes / START | CNN Accelerator |
| RISC-V | configuration writes / START | Distance/Anomaly Engine |
| RISC-V | configuration writes / START | DMA Controller |
| CNN | BUSY/DONE/ERROR + ACCEL_DONE_IRQ | RISC-V |
| Distance Engine | BUSY/DONE/ERROR + SCORE_DONE_IRQ | RISC-V |
| DMA | BUSY/DONE/ERROR + DMA_DONE_IRQ | RISC-V |
| RISC-V | status reads | all project IPs |

## 4.1 High-Level Control Sequence

Initialize/reset the SoC and accelerator blocks.

RISC-V configures CNN base addresses and dimension parameters.

RISC-V configures DMA source, destination, length and control.

DMA transfers image/weights into the CNN-accessible memory/data path.

RISC-V starts CNN processing.

CNN executes convolution → pooling → embedding generation.

Generated embedding is stored in DMEM.

RISC-V configures the distance/anomaly engine with query/reference addresses, count, metric and threshold.

RISC-V starts the distance engine.

Distance engine computes distances, selects minimum distance and performs threshold comparison.

RISC-V receives SCORE_DONE_IRQ or polls status and reads the result.

Result is reported through software/UART and optionally GPIO.

# 5. Data Path

The data path is separate from register control. Image and weight data are INT8. CNN processing produces a fixed-length embedding. The distance engine consumes the generated/query embedding and stored reference embeddings and produces the minimum distance and MATCH/ANOMALY decision.

| Stage | Input | Operation | Output |
| --- | --- | --- | --- |
| Input Buffer | Input image INT8 | Buffer/format input samples | Buffered image |
| Convolution | Image + CNN weights | INT8 MAC / convolution | Feature maps |
| Pooling | Feature maps | Pooling | Reduced feature maps |
| Embedding Buffer | Pooled features | Pack/store fixed-length representation | Generated embedding INT8 |
| Distance Input | Query + reference embeddings | Fetch vectors | Vector pairs |
| Distance Calculation | Query/reference vectors | Configured distance metric | Distance per reference |
| Minimum Distance | Distances | Select smallest | Minimum distance |
| Threshold Comparison | Minimum distance + threshold | Compare | MATCH / ANOMALY |

## 5.1 DMEM Data Organization

DMEM is 64 KB and stores the principal data objects. Exact offsets inside DMEM are implementation-defined unless fixed by the RTL team; the following logical regions are recommended.

| Logical region | Contents | Type |
| --- | --- | --- |
| Input Image | Grayscale image samples | INT8 |
| CNN Weights | Convolution/filter weights | INT8 |
| Reference Embeddings | Previously stored embeddings | INT8 |
| Generated/Query Embedding | CNN output used by scoring engine | INT8 |

# 6. Register Map

The following register offsets are proposed for implementation to make the specification complete. They are offsets within each IP's 4-KB region. If faculty-provided IP templates already define different offsets, preserve the provided offsets and update the RTL/software consistently.

## 6.1 CNN Accelerator Registers — Base 0x2000_3000

| Offset | Register | R/W | Description |
| --- | --- | --- | --- |
| 0x00 | CTRL | R/W | bit0 CNN_START; bit1 CNN_RESET; optional interrupt enable bit |
| 0x04 | STATUS | R | bit0 CNN_BUSY; bit1 CNN_DONE; bit2 CNN_ERROR |
| 0x08 | IMAGE_BASE_ADDR | R/W | DMEM/input image base address |
| 0x0C | WEIGHT_BASE_ADDR | R/W | CNN weight base address |
| 0x10 | EMBED_BASE_ADDR | R/W | Generated embedding destination address |
| 0x14 | DIM_CFG | R/W | Image/channel/dimension configuration |
| 0x18 | EMBED_LEN | R/W | Embedding length, if required by implementation |
| 0x1C | ERROR_CODE | R | CNN error/status code |

## 6.2 Distance/Anomaly Engine Registers — Base 0x2000_4000

| Offset | Register | R/W | Description |
| --- | --- | --- | --- |
| 0x00 | CTRL | R/W | bit0 DIST_START; bit1 DIST_RESET; optional interrupt enable |
| 0x04 | STATUS | R | bit0 DIST_BUSY; bit1 DIST_DONE; bit2 DIST_ERROR |
| 0x08 | EMBED_BASE_ADDR | R/W | Query/generated embedding address |
| 0x0C | REF_BASE_ADDR | R/W | Reference embedding base address |
| 0x10 | REF_COUNT | R/W | Number of reference embeddings |
| 0x14 | METRIC | R/W | Distance metric selection |
| 0x18 | THRESHOLD | R/W | Programmable anomaly threshold |
| 0x1C | RESULT | R | MATCH/ANOMALY result and/or minimum-distance status |
| 0x20 | MIN_DISTANCE | R | Minimum calculated distance |
| 0x24 | ERROR_CODE | R | Distance-engine error/status code |

## 6.3 DMA Controller Registers — Base 0x2000_5000

| Offset | Register | R/W | Description |
| --- | --- | --- | --- |
| 0x00 | CTRL | R/W | bit0 DMA_START; bit1 DMA_RESET; bit2 DMA_IRQ_EN |
| 0x04 | STATUS | R | bit0 DMA_BUSY; bit1 DMA_DONE; bit2 DMA_ERROR |
| 0x08 | SRC_ADDR | R/W | Source memory address |
| 0x0C | DST_ADDR | R/W | Destination memory address |
| 0x10 | LENGTH | R/W | Transfer length in bytes |
| 0x14 | ERROR_CODE | R | DMA error/status code |

## 6.4 Standard Peripheral Regions

| Peripheral | Base | Size | Register details |
| --- | --- | --- | --- |
| IMEM | 0x0000_0000 | 64 KB | Faculty-provided memory interface |
| DMEM | 0x1000_0000 | 64 KB | Faculty-provided memory interface |
| UART | 0x2000_0000 | 4 KB | Use faculty-provided UART register definition |
| Timer | 0x2000_1000 | 4 KB | Use faculty-provided Timer register definition |
| GPIO | 0x2000_2000 | 4 KB | Use faculty-provided GPIO register definition |

# 7. Signal List

Signal names below are logical project-level signals. Exact RTL port names shall follow the faculty-provided RISC-V/interconnect templates where applicable.

| Category | Signal | Dir. | Width | Purpose |
| --- | --- | --- | --- | --- |
| Clock/Reset | clk | I | 1 | System clock |
| Clock/Reset | rst | I | 1 | Synchronous/asynchronous reset according to SoC convention |
| Bus | addr | I | 32 | Memory-mapped address |
| Bus | write | I | 1 | Write transaction indication |
| Bus | wdata | I | 32 | Write data |
| Bus | wstrb | I | 4 | Byte write enables |
| Bus | valid | I | 1 | Request valid |
| Bus | ready | O | 1 | Request accepted |
| Bus | rdata | O | 32 | Read data |
| Bus | rvalid | O | 1 | Read response valid |
| Bus | rready | I | 1 | Read response accepted |
| Bus | error | O | 1 | Transaction/error response |
| Arbitration | bus_req | I | 1 | Master requests bus |
| Arbitration | bus_grant | O | 1 | Arbiter grants bus |
| CNN | cnn_start | I | 1 | Start CNN operation |
| CNN | cnn_busy | O | 1 | CNN processing active |
| CNN | cnn_done | O | 1 | CNN operation completed |
| CNN | cnn_error | O | 1 | CNN error |
| CNN | accel_done_irq | O | 1 | CNN completion interrupt |
| Distance | dist_start | I | 1 | Start scoring |
| Distance | dist_busy | O | 1 | Scoring active |
| Distance | dist_done | O | 1 | Scoring completed |
| Distance | dist_error | O | 1 | Distance-engine error |
| Distance | score_done_irq | O | 1 | Scoring completion interrupt |
| DMA | dma_start | I | 1 | Start DMA transfer |
| DMA | dma_busy | O | 1 | DMA transfer active |
| DMA | dma_done | O | 1 | DMA transfer completed |
| DMA | dma_error | O | 1 | DMA error |
| DMA | dma_done_irq | O | 1 | DMA completion interrupt |
| DMA | src_addr | I | 32 | DMA source |
| DMA | dst_addr | I | 32 | DMA destination |
| DMA | length | I | 32 | DMA transfer length |

# 8. Interrupt Architecture

Three project-specific completion interrupts are defined:

| Interrupt | Source | Destination | When asserted | CPU action |
| --- | --- | --- | --- | --- |
| ACCEL_DONE_IRQ | CNN Accelerator | RISC-V | CNN embedding generation completes | Read CNN status/result and proceed to scoring |
| SCORE_DONE_IRQ | Distance Engine | RISC-V | Distance and threshold decision completes | Read minimum distance/result |
| DMA_DONE_IRQ | DMA Controller | RISC-V | Configured DMA transfer completes | Check status and continue processing |

Interrupt behavior recommendation: generate a completion pulse or latch a pending bit until the CPU clears/acknowledges it. The exact interrupt-controller implementation is not defined in the source architecture and should follow the faculty-provided RISC-V/SoC environment.

# 9. Error Handling

| Error | Possible source | Detection | Required behavior |
| --- | --- | --- | --- |
| Invalid address | Interconnect | Address decode miss | Return bus ERROR; do not modify a slave |
| Illegal register access | CNN/Distance/DMA | Unsupported offset or access type | Set ERROR status; return error response |
| Busy START | Accelerators/DMA | START written while BUSY=1 | Reject or ignore START; set error/status according to implementation policy |
| Invalid dimension/config | CNN | DIM_CFG validation | Set CNN_ERROR and ERROR_CODE |
| Invalid reference count | Distance Engine | REF_COUNT validation | Set DIST_ERROR |
| Invalid metric | Distance Engine | METRIC validation | Set DIST_ERROR |
| DMA length/address error | DMA | Range/alignment validation | Set DMA_ERROR and stop transfer |
| DMA bus error | DMA/interconnect | Bus response | Abort transfer and latch error |
| Timeout/watchdog | Any long-running IP | Optional cycle counter | Set error and return to idle |

## 9.1 Error Priority

For each IP, ERROR should have priority over DONE for the same operation. Once an error is latched, BUSY shall clear and the IP shall return to an idle/error state until software resets or clears the error according to the implemented policy.

# 10. Reset Behavior

Reset places the RISC-V-visible control/status registers into known default values.

CNN, Distance Engine and DMA enter IDLE with BUSY=0, DONE=0 and ERROR=0.

Pending completion interrupts are cleared on reset.

No DMA transfer shall be active after reset.

Memory contents are not assumed to be initialized by accelerator reset unless explicitly implemented.

Software must configure base addresses, dimensions, metric, reference count and threshold before starting processing.

# 11. Operation / State Machines

| IP | States | Transition summary |
| --- | --- | --- |
| CNN | IDLE → LOAD/WAIT → CONV → POOL → EMBED_WRITE → DONE/ERROR | START launches operation; completion raises DONE/IRQ |
| Distance Engine | IDLE → LOAD_QUERY → LOAD_REF → DISTANCE → MIN_SELECT → THRESHOLD → DONE/ERROR | Processes configured reference count |
| DMA | IDLE → ARBITRATE → READ → WRITE → COMPLETE/ERROR | Transfers LENGTH bytes with address increment |

# 12. Detailed End-to-End Data Flow

Input image, CNN weights and reference embeddings reside in DMEM or are placed there before accelerator execution.

DMA may transfer the input image and CNN weights to the CNN input/data interface as configured.

CNN reads the image/weights and executes INT8 convolution.

Pooling reduces the feature representation.

The embedding buffer stores the fixed-length generated embedding.

Generated embedding is written to DMEM at EMBED_BASE_ADDR.

Distance engine reads the generated/query embedding and reference embeddings.

For each reference, the configured distance metric is calculated.

The minimum distance is selected.

Minimum distance is compared with THRESHOLD.

The engine outputs MATCH or ANOMALY and asserts SCORE_DONE_IRQ.

RISC-V reads the result and can report it through UART/GPIO.

# 13. Software / Firmware Responsibilities

Initialize memory/peripherals and clear accelerator status.

Program CNN register set.

Program DMA source/destination/length and start DMA where required.

Wait for DMA completion using IRQ or polling.

Start CNN and wait for ACCEL_DONE_IRQ or CNN_DONE polling.

Program distance-engine query/reference addresses, reference count, metric and threshold.

Start distance engine.

Handle SCORE_DONE_IRQ or poll DIST_STATUS.

Read RESULT and MIN_DISTANCE.

Transmit final result through UART and/or drive GPIO.

# 14. Verification Plan

| Test area | Test | Expected result |
| --- | --- | --- |
| Interconnect | Read/write each mapped region | Correct target selected |
| Interconnect | Access unmapped address | ERROR response |
| CNN | Known image + weights | Embedding matches Python/NumPy golden model within defined fixed-point behavior |
| CNN | Reset during idle | Returns to clean IDLE |
| DMA | Known source/destination/length | Destination equals source data |
| DMA | Invalid range/length | DMA_ERROR |
| Distance | Known query/reference vectors | Correct distance/minimum |
| Threshold | Distance below/above threshold | Correct MATCH/ANOMALY |
| Interrupts | Complete each IP operation | Exactly corresponding IRQ asserted |
| Integration | End-to-end image test | Expected classification and no bus errors |

# 15. Kiro Implementation Requirements

Kiro should use this document as the primary functional specification and follow these implementation rules:

Do not change the top-level functional architecture without explicitly flagging the change.

Implement the 32-bit memory-mapped system-bus abstraction and a 2-master × 8-slave address-decoding interconnect.

Use the faculty-provided RISC-V core unchanged unless integration requires a documented adapter.

Treat DMA as both a memory-mapped slave for CPU configuration and a secondary bus master for bulk transfers.

Keep CNN datapath and Distance/Anomaly datapath modular and independently testable.

Use INT8 data for image/weights/embeddings as specified.

Keep control/status registers memory mapped.

Implement DONE/ERROR latching and the three named interrupts.

Preserve the memory-map base addresses and region sizes defined in Section 3.

Add assertions for invalid addresses, illegal starts while BUSY, invalid configuration and DMA errors.

Create self-checking testbenches and compare accelerator outputs against Python/NumPy golden-model results.

Where this document marks a register detail as proposed, verify compatibility with faculty-provided templates before final RTL freeze.

# 16. Applications

Embedded image similarity checking.

Visual anomaly detection for small grayscale images.

Industrial inspection and defect screening.

Low-power edge classification and similarity matching.

Embedded sensor/image monitoring systems.

Hardware-accelerated proof-of-concept for RISC-V edge-AI SoCs.

# 17. Design Constraints and Assumptions

The project targets a compact embedded image-processing workload suitable for a two-month academic implementation.

Input/weights/embedding data are INT8.

DMEM capacity is 64 KB.

The CNN is lightweight; exact layer count, dimensions and embedding length must be selected so that the design fits the available memory/resources.

Reference embeddings are stored in DMEM for the baseline implementation.

The distance metric is programmable, but the exact supported metric set must be finalized in RTL; a simple integer metric is preferred for implementation simplicity.

The threshold is programmable.

Exact AXI signal-level adaptation is an implementation detail of the faculty-provided interconnect wrapper and must not alter the logical memory map.

# 18. Final Implementation Checklist

| Item | Status / requirement |
| --- | --- |
| System architecture | RISC-V + 32-bit memory-mapped bus + 3 project IPs + standard peripherals |
| Masters | 2: RISC-V and DMA |
| Slave regions | 8: IMEM, DMEM, UART, Timer, GPIO, CNN, Distance, DMA registers |
| Memory map | Defined in Section 3 |
| Control path | Defined in Section 4 |
| Data path | Defined in Section 5 |
| CNN registers | Defined in Section 6.1 |
| Distance registers | Defined in Section 6.2 |
| DMA registers | Defined in Section 6.3 |
| Signal list | Defined in Section 7 |
| Interrupts | ACCEL_DONE_IRQ, SCORE_DONE_IRQ, DMA_DONE_IRQ |
| Errors | Defined in Section 9 |
| Reset | Defined in Section 10 |
| Verification | Python/NumPy golden model + RTL tests |
| Interconnect generation | python3 axi_interconnect_wrap.py -p 2 8 |

# 19. Source Basis and Change Note

The uploaded specification document establishes the project title, system purpose, three project-specific IP blocks, objectives, and the control/data-path diagrams. In particular, the source document states that the system provides memory-mapped access to peripherals, supports INT8 processing, DMA transfers, interrupt completion and Python/NumPy verification.

The supplied page-3 architecture image labels the bus as a 'System Bus / Interconnect (AXI-Lite style, memory-mapped)' and page-4 labels it as 'SYSTEM BUS (32-bit Memory Mapped)'. For consistency with the current project decision, this specification uses '32-bit Memory-Mapped System Bus' as the conceptual architecture term. AXI terminology is retained only for the faculty-provided RTL interconnect-generation script.